## Competence in automotive and industrial electronics

- Micronas (SIX Swiss Exchange: MASN) is known and recognized in the automotive and industrial business as a reliable global acting partner for intelligent, sensor-based system solutions. Micronas offers a variety of Hall sensors and embedded controllers for smart actuators for automotive and industrial applications, such as drive trains, chassis frames, engine management and convenience functions.

- Micronas serves all major automotive electronics customers worldwide, many of them in continuous partnerships seeking joint success. While the holding company is headquartered in Zurich (Switzerland), operational headquarters are based in Freiburg (Germany). Currently, the Micronas Group employs around 900 people. For more information about Micronas and its products, please visit www.micronas.com.

# Hall-effect sensors

Micronas supplies a wide range of hall-effect sensors for many different kinds of applications. Micronas has pioneered CMOS based hall-effect sensors offering today highest quality products for different parameters like position, pressure, speed, level and current. Different sensor families have been developed in the past. The switch family includes simple switches (HAL 1xy / HAL 2xy, HAL 5xy), double-plate switches (HAL 3xy and HAL 7xy) as well as highly complex programmable switches (HAL 1000). Another major family is comprised out of linear sensors which includes non-programmable (HAL 4xy) as well as programmable (HAL 8xy) versions but also with direct network interfacing like LIN (HAL 28xy). The latter represents a so-called "smart or intelligent sensor" which means a further step on system integration level including already a small microcontroller.

By means of Micronas’ 3D-HAL sensor technology (HAL 36xy/HAL 38xy) the Direct Angle product family represents a new level of performance for Hall-effect sensors. Due to their additional vertical Hall plates the sensors enable rotational movement detection up to 360° and contribute to a significant simplification in the design of magnetic systems.

Micronas today offers the world’s broadest range of Hall-effect sensors. Our expertise in Hall sensors combines perfectly with our CMOS and mixed-signal design resources to create accurate, intelligent sensors for a broad range of Automotive and Industrial applications.

In Automotive electronics, Micronas is focusing on sensor systems for a wide range of applications. They require on one hand rather simple Hall switches to detect a position (e.g. seat position, door lock, seatbelt) and on the other hand quite complex linear Hall-effect sensors for the measurement of a travel distance (e.g. clutch or brake pedal, adaptive front light) or of a rotational movement (throttle position, general purpose actuator).

Hall sensors are increasingly establishing themselves in automated manufacturing processes and household equipment. Micronas' sensor solutions are in demand for industrial applications such as robotics controllers and systems to automate manufacturing equipment. Hall sensors can also be used in many household items, including washing machines, tumble driers, cooker hobs, and heating and cooling systems.

Hall-effect sensors for:

- Linear movement measurement

- Direct angle detection

- Position and direction detection

- Current measurement

- Pressure measurement

- Rotation speed measurement

# Embedded microcontrollers/smart actuators

In developing its products, Micronas can draw on more than 30 years' automotive experience, going right back to the first electronic circuitry employed in the car interior - the car clock IC. The increasing functionality over the past years of Micronas semiconductor products has led to the major product family of actuator system solutions / embedded microcontrollers.

Due to their high integration level these high-voltage controllers (HVC 24xyB) feature a compact and cost-effective system design for the use in automotive and industrial applications. Various integrated digital and analog building blocks such as comparators with virtual star point reference or embedded amplifier allow users to minimize the number of external components. Beside timers/ counters, interrupt controller, multichannel A/D converter and enhanced PWMs, this family contains a voltage regulator for direct 12 V supply and a LIN physical layer interface. This makes the system lighter in weight and saves important space within the application.

Smart actuator system solutions for:

- DC motor drives

- Fans

- Pumps