Excellence in Automotive and Industrial Electronics

Micronas presents the GAS 86xyB – the smallest and most efficient multi-parametric sensor platform for gas detection

- 50% reduced power consumption and minimized size compared to its predecessor

- Simultaneous detection of two gases and integrated measurement of ambient temperature and humidity

- Factory-calibration of the temperature and humidity sensors

- More powerful digital signal processing

- Applications: Early fire detection, iAQ/AQS, HVAC systems, trace gases / leakage detection

- Open QFN package with integrated PTFE filter

- Product information on GAS 86xyB

Micronas sets new standards for linear Hall-effect sensors with its new HAL 24xy family

- Very high precision due to 16-bit signal path with integrated DSP-core

- Linearization of the output signal with via 16 programmable setpoints

- Minimized system costs, smaller magnets, increased distance measurements

- Angular measurements up to 180°

- Very high ESD protection with 8 kV (HBM)

- Various diagnostic functions like internal self-tests of the signal path and memory environment

- Development-kit available

- Applications: Throttle valve position, accelerator position, EGR

- Package: TO92UT

- Product information on HAL 24xy

Micronas set to enlarge its 3D Hall sensor family

- Further investment into the development of HAL 36xy and HAL 38xy families enlarges Micronas' product portfolio in the field of multi-dimensional magnetic field detection

Via vertical Hall plates, Micronas' 3D HAL technology enables the detection magnetic field components running parallel to the surface

License Note

HAL 36xy/38xy uses licenses of Fraunhofer Institute for Integrated Circuits IIS.

- Detection of angles greater than 90° or measurement of linear movements with distances of as much as 40 mm

- HAL 36xy for highly precise angular measurement between 0° and 360°

- HAL 38xy for detecting linear motions with ultimate accuracy

- Application examples: contactless potentiometers or position determination in rotary movements (HAL 3625), EGR throttle position detection (HAL 3855)

- SOIC8 SMD package

- Product information on HAL 3625 and HAL 3855

Micronas now offers the successful HAL 880 linear Hall sensor in SMD package

- The HAL 880 in SOIC8 combines high-performance sensor technology with a cost-effective, manufacturing-friendly package, leading to attractive cost improvements at the system level.

- The HAL 880 is a member of Micronas’ well-known linear Hall sensor family, which has over 500 million units in production. Other family members will be available in SOIC8 packaging in the future.

- The sensor is equipped with a linear analog output and is particularly suitable for applications where linear movements (distance, position) have to be detected, e.g. for factory automation and for consumer white-goods.

- Product information on HAL 880

Micronas presenting new generation of Hall sensors for extended position measurement

- The HAL 38xy family is firmly based on the 3D Hall technology which features two vertical and one horizontal Hall plate

- Accompanying Micronas’ market launch of this new generation is a first spin-off, the HAL 3855

The HAL 3855 is strictly speaking a 2D position sensor, as it needs only two components of a magnetic field in order to make a linear position measurement

License Note

HAL 36xy/38xy uses licenses of Fraunhofer Institute for Integrated Circuits IIS.

- As the displacement or position information gained is on principle subject to variations, the HAL 3855 has the ability to linearize the output signal using 32 support points with 16 bit resolution

- The HAL 3855 features numerous diagnosis functions such as over- and under-voltage detection, and it is capable of detecting a break in the supply lines or the loss of the magnet

Micronas current transducer family with digital output for automotive and industrial applications

- With the CUR 315x family Micronas expands its range of sensors beyond the existing CUR 3105 for contactless current measurement applications which benefit from the combination of high accuracy and digital output signal.

- Current measurements from the milliampere range up to hundreds of amperes at static and alternating magnetic fields up to 2 kHz.

- Sensitivity error below 1 percent under optimum conditions

- Operable at magnetic fields from ±20 to ±160 mT.

- Suitable for applications, such as battery monitoring, e.g. in hybrid or electric vehicles, in start-stop systems, smart metering and solar inverters.

- Product information on CUR 315x

- Product information on GAS 85xyB

Company

- Company backgrounder

- Contacts for members of the press

- Company presentation